- CIS 83 LAB 5 - VLAN

- Rich Simms

- October 16, 2006

- Objective

- The objective of this lab is to become familiar with configuring VLANs, VTP, trunking, ports and using a router for inter-VLAN routing.

- Scenario

- This lab was done using the basic router pod on NETLAB and the Cape Town pod in the lab.  NETLAB lets you remotely access a pod of Cisco switches.  The two Cisco switches were configured as shown below.

- Topology

- Final Running Configurations

| - 2950 | - 2900XL | - Router |
| --- | --- | --- |
| 2950#show run Building configuration... Current configuration : 2496 bytes ! version 12.1 no service pad service timestamps debug uptime service timestamps log uptime no service password-encryption ! hostname 2950 ! ! ip subnet-zero ! ! spanning-tree mode pvst no spanning-tree optimize bpdu transmission spanning-tree extend system-id ! ! interface FastEthernet0/1 switchport mode trunk no ip address ! interface FastEthernet0/2 switchport mode trunk no ip address ! interface FastEthernet0/3 no ip address ! interface FastEthernet0/4 no ip address ! interface FastEthernet0/5 switchport access vlan 10 switchport mode access no ip address ! interface FastEthernet0/6 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/7 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/8 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/9 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/10 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/11 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/12 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/13 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/14 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/15 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/16 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/17 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/18 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/19 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/20 switchport access vlan 20 switchport mode access no ip address ! interface FastEthernet0/21 switchport access vlan 30 switchport mode access no ip address ! interface FastEthernet0/22 no ip address ! interface FastEthernet0/23 no ip address ! interface FastEthernet0/24 switchport mode access no ip address ! interface Vlan1 ip address 172.16.1.2 255.255.255.0 no ip route-cache shutdown ! ip default-gateway 172.16.1.1 ip http server ! ! line con 0 exec-timeout 0 0 logging synchronous line vty 0 4 password cisco login line vty 5 15 login ! end 2950# | 2900XL#show run Building configuration... Current configuration : 1443 bytes ! version 12.1 no service pad service timestamps debug uptime service timestamps log uptime no service password-encryption ! hostname 2900XL ! ! ip subnet-zero ! ! spanning-tree mode pvst no spanning-tree optimize bpdu transmission spanning-tree extend system-id ! ! ! ! interface FastEthernet0/1 ! interface FastEthernet0/2 switchport mode trunk ! interface FastEthernet0/3 ! interface FastEthernet0/4 ! interface FastEthernet0/5 ! interface FastEthernet0/6 ! interface FastEthernet0/7 ! interface FastEthernet0/8 ! interface FastEthernet0/9 ! interface FastEthernet0/10 switchport access vlan 20 switchport mode access ! interface FastEthernet0/11 switchport access vlan 20 switchport mode access ! interface FastEthernet0/12 switchport access vlan 20 switchport mode access ! interface FastEthernet0/13 ! interface FastEthernet0/14 ! interface FastEthernet0/15 ! interface FastEthernet0/16 ! interface FastEthernet0/17 ! interface FastEthernet0/18 ! interface FastEthernet0/19 ! interface FastEthernet0/20 ! interface FastEthernet0/21 ! interface FastEthernet0/22 ! interface FastEthernet0/23 ! interface FastEthernet0/24 ! interface GigabitEthernet0/1 ! interface GigabitEthernet0/2 ! interface Vlan1 ip address 172.16.1.3 255.255.255.0 no ip route-cache shutdown ! ip default-gateway 172.16.1.1 ip http server ! line con 0 exec-timeout 0 0 logging synchronous line vty 0 4 password cisco login line vty 5 15 login ! ! end 2900XL# | Router#show run Building configuration... Current configuration : 1093 bytes ! version 12.2 service timestamps debug uptime service timestamps log uptime no service password-encryption ! hostname Router ! ! memory-size iomem 15 ip subnet-zero ! ! ! call rsvp-sync ! ! ! ! ! ! controller T1 1/0 framing sf linecode ami ! ! ! interface FastEthernet0/0 no ip address duplex auto speed auto ! interface FastEthernet0/0.1 encapsulation dot1Q 1 native ip address 172.16.1.1 255.255.255.0 ! interface FastEthernet0/0.10 encapsulation dot1Q 10 ip address 172.16.10.1 255.255.255.0 ! interface FastEthernet0/0.20 encapsulation dot1Q 20 ip address 172.16.20.1 255.255.255.0 ! interface FastEthernet0/0.30 encapsulation dot1Q 30 ip address 172.16.30.1 255.255.255.0 ! interface Serial0/0 no ip address shutdown no fair-queue ! interface BRI0/0 no ip address encapsulation hdlc shutdown ! interface FastEthernet0/1 no ip address shutdown duplex auto speed auto ! interface Serial0/1 no ip address shutdown ! ip classless ip http server ! ! dial-peer cor custom ! ! ! ! line con 0 exec-timeout 0 0 logging synchronous line aux 0 line vty 0 4 login ! end Router# |

Additional Outputs

| 2950 | 2950#show vlan VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa0/4, Fa0/22, Fa0/23 Fa0/24 10 Accounting active Fa0/5 20 Marketing active Fa0/6, Fa0/7, Fa0/8, Fa0/9 Fa0/10, Fa0/11, Fa0/12, Fa0/13 Fa0/14, Fa0/15, Fa0/16, Fa0/17 Fa0/18, Fa0/19, Fa0/20 30 Engineering active Fa0/21 1002 fddi-default act/unsup 1003 token-ring-default act/unsup 1004 fddinet-default act/unsup 1005 trnet-default act/unsup VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2 ---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------ 1 enet 100001 1500 - - - - - 0 0 10 enet 100010 1500 - - - - - 0 0 20 enet 100020 1500 - - - - - 0 0 30 enet 100030 1500 - - - - - 0 0 1002 fddi 101002 1500 - - - - - 0 0 VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2 ---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------ 1003 tr 101003 1500 - - - - - 0 0 1004 fdnet 101004 1500 - - - ieee - 0 0 1005 trnet 101005 1500 - - - ibm - 0 0 Remote SPAN VLANs ------------------------------------------------------------------------------ Primary Secondary Type Ports ------- --------- ----------------- ------------------------------------------ 2950#2950#show vtp status VTP Version : 2 Configuration Revision : 4 Maximum VLANs supported locally : 64 Number of existing VLANs : 8 VTP Operating Mode : Server VTP Domain Name : group1 VTP Pruning Mode : Disabled VTP V2 Mode : Disabled VTP Traps Generation : Disabled MD5 digest : 0xAF 0x65 0x97 0x40 0xDD 0x4B 0xCE 0xE0 Configuration last modified by 172.16.1.2 at 3-1-93 00:26:16 Local updater ID is 172.16.1.2 on interface Vl1 (first interface found) 2950# 2950#show int fa 0/5 switchport Name: Fa0/5 Switchport: Enabled Administrative Mode: static access Operational Mode: down Administrative Trunking Encapsulation: dot1q Negotiation of Trunking: Off Access Mode VLAN: 10 (Accounting) Trunking Native Mode VLAN: 1 (default) Voice VLAN: none Administrative private-vlan host-association: none Administrative private-vlan mapping: none Administrative private-vlan trunk native VLAN: none Administrative private-vlan trunk encapsulation: dot1q Administrative private-vlan trunk normal VLANs: none Administrative private-vlan trunk private VLANs: none Operational private-vlan: none Trunking VLANs Enabled: ALL Pruning VLANs Enabled: 2-1001 Capture Mode Disabled Capture VLANs Allowed: ALL Protected: false Appliance trust: none 2950#show int fa 0/2 switchport Name: Fa0/2 Switchport: Enabled Administrative Mode: trunk Operational Mode: trunk Administrative Trunking Encapsulation: dot1q Operational Trunking Encapsulation: dot1q Negotiation of Trunking: On Access Mode VLAN: 1 (default) Trunking Native Mode VLAN: 1 (default) Voice VLAN: none Administrative private-vlan host-association: none Administrative private-vlan mapping: none Administrative private-vlan trunk native VLAN: none Administrative private-vlan trunk encapsulation: dot1q Administrative private-vlan trunk normal VLANs: none Administrative private-vlan trunk private VLANs: none Operational private-vlan: none Trunking VLANs Enabled: ALL Pruning VLANs Enabled: 2-1001 Capture Mode Disabled Capture VLANs Allowed: ALL Protected: false Appliance trust: none 2950# |
| --- | --- |
| 2900XL | 2900XL#show vlan VLAN Name Status Ports ---- -------------------------------- --------- ------------------------------- 1 default active Fa0/1, Fa0/4, Fa0/5, Fa0/6 Fa0/7, Fa0/8, Fa0/9, Fa0/13 Fa0/14, Fa0/15, Fa0/16, Fa0/17 Fa0/18, Fa0/19, Fa0/20, Fa0/21 Fa0/22, Fa0/23, Fa0/24, Gi0/1 Gi0/2 10 Accounting active 20 Marketing active Fa0/10, Fa0/11, Fa0/12 30 Engineering active 1002 fddi-default act/unsup 1003 token-ring-default act/unsup 1004 fddinet-default act/unsup 1005 trnet-default act/unsup VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2 ---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------ 1 enet 100001 1500 - - - - - 0 0 10 enet 100010 1500 - - - - - 0 0 20 enet 100020 1500 - - - - - 0 0 30 enet 100030 1500 - - - - - 0 0 VLAN Type SAID MTU Parent RingNo BridgeNo Stp BrdgMode Trans1 Trans2 ---- ----- ---------- ----- ------ ------ -------- ---- -------- ------ ------ 1002 fddi 101002 1500 - - - - - 0 0 1003 tr 101003 1500 - - - - srb 0 0 1004 fdnet 101004 1500 - - - ieee - 0 0 1005 trnet 101005 1500 - - - ibm - 0 0 Remote SPAN VLANs ------------------------------------------------------------------------------ Primary Secondary Type Ports ------- --------- ----------------- ------------------------------------------ 2900XL#show vtp status VTP Version : 2 Configuration Revision : 4 Maximum VLANs supported locally : 250 Number of existing VLANs : 8 VTP Operating Mode : Client VTP Domain Name : group1 VTP Pruning Mode : Disabled VTP V2 Mode : Disabled VTP Traps Generation : Disabled MD5 digest : 0xAF 0x65 0x97 0x40 0xDD 0x4B 0xCE 0xE0 Configuration last modified by 172.16.1.2 at 3-1-93 00:26:16 2900XL# 2900XL# show int fa 0/2 switchport Name: Fa0/2 Switchport: Enabled Administrative Mode: trunk Operational Mode: trunk Administrative Trunking Encapsulation: dot1q Operational Trunking Encapsulation: dot1q Negotiation of Trunking: On Access Mode VLAN: 1 (default) Trunking Native Mode VLAN: 1 (default) Voice VLAN: none Administrative private-vlan host-association: none Administrative private-vlan mapping: none Administrative private-vlan trunk native VLAN: none Administrative private-vlan trunk encapsulation: dot1q Administrative private-vlan trunk normal VLANs: none Administrative private-vlan trunk private VLANs: none Operational private-vlan: none Trunking VLANs Enabled: ALL Pruning VLANs Enabled: 2-1001 Capture Mode Disabled Capture VLANs Allowed: ALL Protected: false Unknown unicast blocked: disabled Unknown multicast blocked: disabled Appliance trust: none 2900XL# show int fa 0/5 switchport Name: Fa0/5 Switchport: Enabled Administrative Mode: dynamic desirable Operational Mode: static access Administrative Trunking Encapsulation: dot1q Operational Trunking Encapsulation: native Negotiation of Trunking: On Access Mode VLAN: 1 (default) Trunking Native Mode VLAN: 1 (default) Voice VLAN: none Administrative private-vlan host-association: none Administrative private-vlan mapping: none Administrative private-vlan trunk native VLAN: none Administrative private-vlan trunk encapsulation: dot1q Administrative private-vlan trunk normal VLANs: none Administrative private-vlan trunk private VLANs: none Operational private-vlan: none Trunking VLANs Enabled: ALL Pruning VLANs Enabled: 2-1001 Capture Mode Disabled Capture VLANs Allowed: ALL Protected: false Unknown unicast blocked: disabled Unknown multicast blocked: disabled Appliance trust: none 2900XL# |
| Router | Router#show ip route Codes: C - connected, S - static, I - IGRP, R - RIP, M - mobile, B - BGP D - EIGRP, EX - EIGRP external, O - OSPF, IA - OSPF inter area N1 - OSPF NSSA external type 1, N2 - OSPF NSSA external type 2 E1 - OSPF external type 1, E2 - OSPF external type 2, E - EGP i - IS-IS, su - IS-IS summary, L1 - IS-IS level-1, L2 - IS-IS level-2 ia - IS-IS inter area, * - candidate default, U - per-user static route o - ODR, P - periodic downloaded static route Gateway of last resort is not set 172.16.0.0/24 is subnetted, 4 subnets C 172.16.30.0 is directly connected, FastEthernet0/0.30 C 172.16.20.0 is directly connected, FastEthernet0/0.20 C 172.16.10.0 is directly connected, FastEthernet0/0.10 C 172.16.1.0 is directly connected, FastEthernet0/0.1 Router#show ip int brief Interface IP-Address OK? Method Status Prot ocol FastEthernet0/0 unassigned YES manual up up FastEthernet0/0.1 172.16.1.1 YES manual up up FastEthernet0/0.10 172.16.10.1 YES manual up up FastEthernet0/0.20 172.16.20.1 YES manual up up FastEthernet0/0.30 172.16.30.1 YES manual up up Serial0/0 unassigned YES manual administratively down down BRI0/0 unassigned YES manual administratively down down BRI0/0:1 unassigned YES unset administratively down down BRI0/0:2 unassigned YES unset administratively down down FastEthernet0/1 unassigned YES manual administratively down down Serial0/1 unassigned YES manual administratively down down Router# |

- Summary

- The Cisco switches are highly customizable.  In this lab we configured VLANs, utilized VTP, set up trunking and did inter-VLAN routing with a router.

- This lab was done on NetLab and on the Cape Town pod in the lab.  For the pod it was necessary to use a cross-over cable between the switches.

- VLANs let you group ports together.  Packets stay within a VLAN and do not flow to other VLANs just as if they were on separate switches.  The configuration was straightforward in NetLab however on the Cape Town lab pod some minor IOS differences were observed between a 2900XL and a 2950.  For example the 2950 supported port ranges and the 2950XL did not.  The 2900XL had to be configured one port at a time.

- VTP was straightforward in NetLab and the Pod.  This is a very useful feature for having consistent VLAN naming across multiple switches from a single database of information.

- Trunking performed as advertised and all packets going through the trunk were correctly channeled back into their respective VLANs on the other end.  Ports were manually configured to be trunked rather rely on default settings.

- Inter-VLAN routing allows traffic between VLANs using a router.  This worked fine in NetLab but would not initially work on the Cape Town pod.  The router had an old version of IOS that did not support the encapsulation command on the sub-interfaces.  After updating the router to v12.2 the encapsulation dot1q command was available and inter-VLAN routing became possible.

Commands

! Clearing out a switch (privilege mode)

clear port-security dynamic

delete flash:vlan.dat

erase startup-config

reload

! Configuring VTP client (privilege mode)

vlan database

vtp client

vtp domain corporate

vtp password cisco

exit

! Configuring VTP server (privilege mode)

vlan database

vtp server

vtp domain corporate

vtp password cisco

vlan 10 name accounting

vlan 20 name marketing

vlan 30 name engineering

exit

! Configuring access ports on 2950

int range fa 0/19 - 24

switchport mode access

switchport access vlan 30

! Configuring access ports on 2900XL

int fa 0/19

switchport mode access

switchport access vlan 30

! Configuring trunks on 2900xl

int fa 0/1

switchport trunk encaps dot1q

switchport mode trunk

! Configuring trunks on 2950

int fa 0/1

switchport mode trunk

! Switch management network settings

int vlan1

ip address 17.16.1.2 255.255.255.0

no shut

!

ip default-gateway 172.16.1.1

! Verify switch configurations

show run

show vtp status

show vlan

show spanning-tree

show int fa 0/1 switchport

! Configure sub-interfaces on a router

int fa0/0

no shut

int fa0/0.1

encaps dot1Q 1 native

ip address 172.16.1.1 255.255.255.0

no shut

int fa0/0.10

encaps dot1Q 10

ip address 172.16.10.1 255.255.255.0

no shut

int fa0/0.20

encaps dot1Q 20

ip address 172.16.20.1 255.255.255.0

no shut

int fa0/0.30

encaps dot1Q 30

ip address 172.16.30.1 255.255.255.0

no shut